Introduction

This report provides an overview of my internship experience at a transformer manufacturing company. It highlights the key processes involved in the production of electrical transformers, including core cutting, coil winding, core-coil assembly, oil filling, and final testing. The internship offered valuable practical exposure to industrial machinery, safety protocols, and quality control methods used in modern transformer manufacturing.

Core Cutting Machine – Overview

A Core Cutting Machine is a precision machine used to cut CRGO (Cold Rolled Grain Oriented) steel sheets into specific shapes and sizes to form the core laminations of transformers. These cores are critical for the transformer's magnetic circuit and efficient energy transfer.

⚙ Types of Core Cutting Machines

Step Lap Core Cutting Machine

Produces step-lap joints for improved flux distribution and lower core loss.

Rectangular Core Cutting Machine

Cuts straight limbs for rectangular core designs.

Toroidal Core Cutting Machine

Specially for toroidal transformers.

V Notch & Hole Punching Core Cutting Machine

Performs cutting along with hole punching for bolt and assembly fitment.

🔧 Working Principle

The machine uses servo-controlled blades or rotary shears to cut CRGO sheets. Based on pre-set dimensions, the CNC system controls the cutting operation with high precision.

Key Processes:

Feeding – CRGO coil is unwound and fed into the machine.

Measuring & Marking – Dimensions are calculated via sensors or programmed logic.

Cutting – Hydraulic or servo blades make angular or straight cuts.

Notching – Holes or V-notches are punched where needed.

Stacking – Finished laminations are stacked or discharged automatically.

🧩 Main Components

Component	Function

Uncoiler	Holds and unwinds CRGO coil

Straightener	Flattens the steel before cutting

Servo Feeder	Feeds material precisely to cutting area

Cutting Blade/Tool	Cuts material based on CNC input

Punching Unit	Punches holes or notches for assembly

CNC Controller	Controls the machine operation digitally

Stacker or Conveyor	Collects finished laminations after cutting

📐 Types of Cuts Made

45° or 135° Bevel Cuts

90° Straight Cuts

Step-lap Cuts

Mitred Corners

V-Notch and Bolt Holes

✅ Advantages

High precision and repeatability

Reduces material wastage

Increases efficiency and speed

Improves transformer performance via accurate core design

Reduces no-load losses in transformer core

🛠 Applications

Power Transformers

Distribution Transformers

Instrument Transformers

Toroidal and Rectangular Core Transformers

Slitting Machine – Overview

A Slitting Machine is used to cut wide coils of CRGO (Cold Rolled Grain Oriented) steel into narrower strips. These strips are then used in transformer core cutting and assembly. It is one of the first steps in core fabrication and ensures the required width and edge quality for further processing.

⚙ Purpose in Transformer Manufacturing

Prepare CRGO steel sheets in specific widths for use in core cutting machines.

Maintain accuracy in strip width, reduce burrs, and ensure clean edges.

🔧 Working Principle of a Slitting Machine

The slitting machine works by feeding a wide steel coil through a series of rotary slitting knives mounted on arbors. These knives rotate and shear the coil into narrower strips of desired widths.

🔄 Step-by-Step Working Process

Uncoiling

The large CRGO coil is placed on the uncoiler stand.

The coil is unwound in a controlled manner.

Straightening (Optional)

Some machines have a straightener to flatten the coil before slitting.

Slitting

The sheet passes through rotary knives (upper and lower circular blades).

The blades slit the coil into multiple narrow strips.

The distance between knives is adjusted based on required strip widths.

Tensioning/Looping

A tension device or loop pit maintains consistent strip tension.

Recoiling

The slit strips are rewound into individual coils on the recoiler shaft.

Discharge

The finished coils are removed for use in core cutting.

🧩 Main Components of a Slitting Machine

Component	Function

Uncoiler	Holds and unwinds the master coil

Feeder Unit	Controls speed and feeds coil into slitting knives

Slitting Head	Contains rotary knives to slit the coil

Separator Discs	Maintain alignment and prevent strip overlap

Tension Unit	Maintains consistent strip tension

Recoiler	Rewinds the slit strips into coils

Strip Guides	Aligns and guides strips during slitting

Control Panel (PLC/CNC)	Automates and manages the entire slitting process

📐 Output Example

If the input coil is:

Width = 1000 mm

You want strips of 100 mm width

Then, the slitting machine will produce 10 strips of 100 mm width from the same coil.

✅ Advantages of Slitting Machine

High cutting speed and precision

Minimal edge burrs

Consistent strip quality

Efficient material utilization

Automation reduces labor and increases safety

🛠 Applications

CRGO Slitting for Transformer Core

Electrical Steel Preparation

Coil Processing in Steel Industry

Cable and Wire Industry

Core Stacking Table – Overview

A Core Stacking Table is a specialized mechanical workstation used to assemble and align the laminated core sheets (cut from CRGO steel) into a complete transformer core. It ensures correct orientation, alignment, and stability of the core limbs and yokes during the stacking process.

This is a critical step in transformer core manufacturing that affects magnetic performance, loss levels, and noise.

⚙ Purpose in Transformer Manufacturing

Helps in assembling the core limbs and yokes accurately.

Ensures tight stacking of CRGO laminations.

Allows easy insertion of HV and LV coils during core assembly.

Reduces core loss, leakage flux, and improves overall transformer efficiency.

🔧 Working Principle of a Core Stacking Table

The table is designed to support the vertical or horizontal stacking of CRGO laminations. Using hydraulic or mechanical lifting mechanisms, the table lowers or raises parts of the core to allow easy insertion of coils or limb assembly. After stacking, the core is clamped and compacted to minimize air gaps.

🔄 Step-by-Step Working Process

Lamination Preparation

Laminations cut using the core cutting machine are arranged as per core design (E-I or step-lap).

Base Placement

Yoke (horizontal laminations) is placed first on the table.

Vertical Limb Stacking

Laminations are stacked vertically between guiding fixtures.

Multiple sheets are stacked in a staggered or step-lap manner to reduce magnetic loss.

Hydraulic Lift (Optional)

Some tables lower the center portion to insert transformer coils.

Once coils are in place, limbs are passed through the coil windows.

Upper Yoke Placement

Once the limbs are complete, the upper yoke is stacked on top.

Clamping and Final Adjustment

The complete core is clamped and tightened to compress laminations.

Measurements and squareness are checked for accuracy.

Lifting / Tilting

The assembled core may be lifted or tilted (using hydraulics or crane) for further assembly or transport.

🧩 Main Components of Core Stacking Table

Component	Description

Table Frame	Rigid steel platform to support laminations

Guide Rails/Fixers	Ensures proper alignment of laminations

Hydraulic Lift Mechanism	Moves sections of the table up/down to allow coil insertion

Tilting System	Allows tilting the core into vertical position for easier handling

Clamping Mechanism	Compresses laminations to minimize air gaps

Control Panel	Manual or motorized control of lifting, tilting, and clamping

🧪 Types of Cores Assembled

Three-Limb Core

Five-Limb Core

Rectangular or Cruciform Core

Step-Lap Core Assembly

✅ Advantages

Accurate lamination alignment reduces core loss

Easy and safe handling of heavy laminations

Allows coil insertion during stacking

Reduces manual labor and improves precision

Minimizes deformation and gap issues in core

🛠 Applications

Power Transformer Core Assembly

Distribution Transformer Core Stacking

Dry-type Transformer Assembly

Reactor Core Assembly

___________________________________________________________________

Coil Winding Machine – Complete Overview

🔹 What is a Coil Winding Machine?

A Coil Winding Machine is an industrial machine used to wind wire (usually copper or aluminum) into coils of specific shapes and sizes. These coils are essential for electromagnetic devices like transformers, motors, inductors, and generators.

In a transformer, these coils form the primary and secondary windings, which are responsible for transferring energy through electromagnetic induction.

🧭 Working Principle

The basic operation involves:

Holding the coil form (bobbin or core) in place on a rotating spindle.

Feeding the wire through a tension control and guiding system.

Rotating the spindle while guiding the wire in a controlled manner to create precise layers.

The machine can be manual, semi-automatic, or fully automatic (CNC controlled).

For transformers, insulation and layering are critical to ensure electrical strength and thermal performance.

🛠 Main Components and Functions

Component	Function

Spindle/Mandrel	Rotates the coil form (core or bobbin) during winding.

Wire Tensioner	Controls the tension of wire to prevent breaks or slack.

Wire Guide	Accurately guides the wire for uniform winding.

Traverse Mechanism	Moves back and forth to layer wire across the bobbin evenly.

Counter/Encoder	Tracks the number of turns made during winding.

Drive Motor	Powers the rotation of the spindle.

Control Panel/CNC Unit	Controls speed, tension, pitch, layering, and stop-start functions.

Safety Shields	Ensures operator safety during operation.

🔄 Types of Coil Winding Machines

Type	Description

Manual Winding Machine	Operated by hand; used in small workshops or for repairs.

Semi-Automatic Machine	Operator sets parameters; machine does the winding.

Fully Automatic (CNC)	Pre-programmed and automated; ideal for precision and high-volume production.

Horizontal Winding Machine	Used for large power transformer LV/HV coils (especially disk or helical coils).

Vertical Winding Machine	Suitable for cylindrical or circular core coils in large transformers.

⚙ Technical Parameters (Example for CNC Winder)

Parameter	Typical Range

Conductor type	Copper/Aluminum (Round/Rectangular)

Wire diameter range	0.2 mm – 5 mm

Winding speed	Up to 1200 RPM

Coil length	Up to 3000 mm

Max. coil weight	1000 kg or more (depending on the machine)

Programmable features	Number of turns, pitch, layer insulation

📌 Applications in Transformer Manufacturing

Primary and Secondary Winding Coils

HV & LV Coil Winding (Helical, Disc, Layer types)

Toroidal Core Winding

Distribution and Power Transformer Coils

✅ Advantages

High precision and uniform winding

Reduced material waste

High-speed production

Consistent quality

Programmable for complex coil shapes

❌ Challenges / Maintenance Needs

Wire breakage due to improper tensioning

Overheating of motor if overloaded

Precise alignment required to avoid cross-layering

Regular lubrication and calibration needed

Dust buildup in CNC panel must be cleaned

____________________________________________________________________

Core Coil Assembly Station – Detailed Working and Overview

🔹 What is a Core Coil Assembly Station?

The Core Coil Assembly Station is the area where the active parts of the transformer — the core and the wound coils — are assembled into a complete magnetic and electrical structure. This step is vital to the overall functioning of the transformer, as it determines the alignment, structural integrity, and dielectric clearances of internal components.

🧭 Purpose and Function

To assemble transformer cores with HV (High Voltage) and LV (Low Voltage) windings.

To ensure accurate positioning of coils around the core limbs or legs.

To maintain electrical clearances, mechanical strength, and dimensional accuracy.

To support final clamping, insulation fitting, and core stacking for magnetic circuit completion.

🛠 Main Equipment and Tools in Core Coil Assembly Station

Tool/Equipment	Purpose

Core stacking table/platform	For positioning and stacking core laminations or core legs

Hydraulic/Manual lifting tools	To lift and position the heavy coils onto the core

Assembly fixtures and jigs	Ensure coils align properly on the core limbs

Clamping tools/frames	Used to apply uniform pressure and secure the assembled structure

Measuring tools (vernier, micrometers, gap gauge)	To verify insulation gaps and positioning accuracy

Insulation materials	For placing pressboard barriers, spacers, and end insulation

⚙ Working Steps of Core Coil Assembly Process

🔧 Step-by-Step Process:

Preparation of Core

The core limbs (assembled from laminated sheets) are stacked on the core stacking table.

Yokes may also be prepared separately depending on transformer type.

Lifting and Mounting of Coils

Pre-wound and dried LV and HV coils are brought to the station.

Using cranes or hydraulic lifts, coils are carefully lowered over the limbs of the core, usually starting with LV winding followed by HV winding (in concentric design).

Insulation Placement

Insulation spacers, end rings, and axial/radial barriers are placed between core and coil, and between LV and HV windings.

Ensures electrical isolation and maintains creepage and clearance distances.

Yoke Assembly

The top yoke laminations are placed and clamped to complete the magnetic circuit.

Yoke insulation is added during the process.

Core Clamping and Tightening

The whole assembly is clamped tightly using pressure plates and tightening bolts or hydraulic clamping devices.

Ensures mechanical stability during operation and transport.

Lead Connection and Tapping

Coil terminals and tap changer leads are carefully brought out and fixed.

Required bushing adapters or support blocks are installed.

Measurement and Testing

Insulation gaps, core alignment, and coil positioning are verified.

Visual inspection for damage or misalignment.

Resistance and insulation tests may be performed before tanking.

📌 Transformer Types Handled at This Station

Distribution Transformers (Pole mounted or pad mounted)

Power Transformers

Dry-type Transformers (Air cooled or resin cast)

Three-phase and Single-phase Transformers

✅ Advantages of Core Coil Assembly Station

Accurate alignment ensures low noise and vibration in operation

Strong mechanical build prevents internal shifting during transportation

Reduces risk of insulation breakdown due to precision gap management

Assembly under controlled environment ensures quality and consistency

❌ Challenges and Precautions

Challenge	Precaution

Misalignment of coils	Use proper jigs, templates, and skilled handling

Damage to insulation or winding	Use padded tools and non-metallic spacers during handling

Core damage due to improper stacking	Ensure flat stacking surface and avoid bending of laminations

Insufficient clamping	Apply uniform torque/pressure as per design specs

Poor lead arrangement	Mark leads clearly and follow lead diagrams

Insulation Cutting Machine – Detailed Information & Working

🔹 **What is an Insulation Cutting Machine?

An** Insulation Cutting Machine** is a specialized machine used to cut electrical insulation materials (like pressboard, kraft paper, crepe paper, or Nomex) into various shapes and sizes required for insulating components inside a transformer — such as windings, spacers, ducts, barriers, end rings, and supports.

These components ensure electrical isolation, mechanical support, and thermal protection inside the transformer.

⚙ Working Principle of the Insulation Cutting Machine

The machine cuts flexible or semi-rigid insulation materials using manual, semi-automatic, or fully automatic mechanisms. The cutting is done via:

Rotary blades,

Straight shear blades,

Die punching, or

CNC knife/laser cutting,

depending on the type and complexity of the required shape.

The basic working process includes:

Feeding insulation material (sheet or roll).

Measuring and aligning material.

Cutting using blades, dies, or a CNC system.

Collecting or stacking the cut pieces.

🛠 Main Components and Functions

Component	Function

Feeding Mechanism	Feeds insulation material into the machine (manual or motorized rollers).

Cutting Blade/Tool	Cuts the material to the programmed size or shape.

Tension Control System	Maintains uniform tension on the roll/sheet during feeding.

Control Panel	Allows operator to set dimensions, shape, and quantity.

Conveyor or Stacking Area	Receives cut pieces for collection or further assembly.

Sensors and Guides	Ensure precision in length, angle, and position.

🔄 Types of Insulation Cutting Machines

Machine Type	Description

Manual Cutting Machine	Operated by hand, suitable for small workshops.

Semi-Automatic Cutter	Motorized blades with manual feeding or alignment.

Fully Automatic CNC Cutter	Program-controlled, precise cutting of complex shapes (rectangular, arc, notched, etc.).

Rotary Die Cutter	Uses rotary dies to cut insulation from rolls for fast, repetitive tasks.

Laser/Knife CNC Cutter	Advanced machines used for precise, burr-free cutting of detailed insulation profiles.

📐 Technical Specifications (Typical CNC Insulation Cutter)

Parameter	Value (Range)

Material Thickness	0.2 mm – 10 mm

Max Cutting Width	500 mm – 1500 mm

Cutting Speed	Up to 1000 mm/sec

Accuracy	±0.3 mm to ±0.5 mm

Blade Types	Rotary, straight, V-notch, or oscillating

Material Types Supported	Pressboard, crepe paper, kraft, Nomex, PET

Control System	PLC or touchscreen HMI

🧾 Step-by-Step Working Process

1. Material Loading

Sheet or roll of insulation material is mounted.

The machine is calibrated for thickness and type.

2. Parameter Setting

Operator selects the program or enters length, width, cut type, quantity.

3. Feeding & Alignment

Material is fed through rollers and aligned using sensors.

4. Cutting Operation

Blades or dies cut the material as per the set shape and dimension.

5. Collection

Cut pieces are collected manually or stacked automatically.

🧰 Applications in Transformer Manufacturing

LV and HV winding insulation barriers

Layer insulation and end rings

Pressboard blocks and spacers

Duct strips for cooling channels

Cylinder wrapping for coils

Core insulation blocks

✅ Advantages

High cutting precision, consistent quality

Saves time in mass production

Reduces material wastage

Enables cutting of complex shapes with holes, notches, arcs

Minimal human error compared to manual cutting

❌ Challenges and Maintenance

Issue	Solution

Blade wear or dullness	Regular sharpening or replacement

Dust accumulation from materials	Frequent cleaning of machine and filters

Misalignment of sheets	Use auto-alignment and sensor-guided feeding

Material tearing or jamming	Adjust tension and blade pressure according to material thickness

Software glitches (CNC types)	Regular system updates and backup of programs

🛡 Safety Precautions

Use gloves and safety glasses during manual feeding.

Keep hands away from blades while operating.

Ensure emergency stop button is functional.

Regularly check for loose components or electrical faults.

_______________________________________________________________________

Vapour Phase Drying (VPD) Oven & Hot Air Oven – Detailed Working and Comparison

🔹 Purpose of Drying in Transformer Manufacturing

Drying is essential to:

Remove moisture and trapped water vapor from windings, insulation, and core.

Improve dielectric strength of insulation.

Reduce the risk of partial discharge, flashover, or breakdown in service.

Ensure long-term reliability of the transformer.

🔧 1. Vapour Phase Drying (VPD) Oven – Detailed Information

✅ What is VPD?

Vapour Phase Drying is an advanced and efficient method for drying core-coil assemblies using a vaporized solvent (like kerosene or white oil) that condenses on the colder transformer parts, transfers heat, and then evaporates moisture from the insulation.

⚙ Working Principle of VPD Oven

Solvent Heating:

A solvent (hydrocarbon like Dowtherm or white oil) is heated in a boiler to create hot vapor (up to 130–150°C).

Vapour Ingress into Vacuum Chamber:

The vapor enters the vacuum drying chamber containing the transformer core-coil assembly.

Condensation on Cold Surface:

The vapor condenses on the cooler surfaces of the assembly, releasing latent heat.

This heats up the insulation from inside out and evaporates absorbed moisture.

Evaporation and Removal of Moisture:

The released moisture is removed via vacuum pumps or condensate drains.

Repeat Cycles:

The process is repeated in cycles for uniform drying.

🛠 Main Components of VPD Oven

Component	Function

Solvent boiler	Heats solvent to generate vapor

Drying chamber	Vacuum-tight chamber where drying takes place

Vacuum system	Maintains low pressure for evaporation

Heat exchangers/Condensers	Helps in condensation and collection of moisture

Control panel	Controls temperature, pressure, and timing parameters

Safety valves & detectors	Ensure safe operation and prevent explosion/fire risks

📐 Key Parameters

Parameter	Typical Range

Temperature	120°C – 150°C

Vacuum Level	0.5 to 1 mbar (very low pressure)

Drying Time	8 – 24 hours (depends on size)

Solvent Used	White oil / Dowtherm

✅ Advantages of VPD Oven

Faster and uniform drying

Less energy consumption than hot air ovens

Effective for thicker insulation systems

Ensures deep moisture removal

No oxidation of copper due to oxygen-free environment

❌ Challenges

High initial capital cost

Requires trained personnel

Solvent handling requires safety measures (flammable, toxic)

Complex control and monitoring system

🔧 2. Hot Air Oven – Detailed Information

✅ What is a Hot Air Oven?

A Hot Air Oven dries transformer components by circulating heated air inside a closed chamber. The heat removes moisture from insulation through evaporation.

⚙ Working Principle of Hot Air Oven

Heating Elements (electric or oil-based) heat the air inside the chamber.

Circulation Fans distribute hot air uniformly.

The core-coil assembly or insulation materials are placed inside the chamber.

As the hot air comes in contact with the materials, moisture is evaporated.

The moist air is expelled through vents or collected using condensers.

🛠 Main Components of Hot Air Oven

Component	Function

Heating coils	Heats the air inside the oven

Air circulation fan	Ensures uniform temperature distribution

Temperature controller	Maintains and adjusts drying temperature

Exhaust vent	Removes moist air

Insulated chamber	Retains heat and houses the materials

📐 Key Parameters

Parameter	Typical Range

Temperature	80°C – 120°C

Drying Time	24 – 72 hours (slower)

Humidity Removal	Through air vent or desiccant

✅ Advantages of Hot Air Oven

Lower cost and simpler to operate

Easy to install and maintain

Good for small to medium transformers

Suitable for insulation pre-drying

❌ Limitations

Slower than VPD

Not effective for thick windings or large power transformers

Possible uneven drying if airflow is poor

Risk of oxidation of copper at high temperature with open air

📊 Comparison: VPD Oven vs Hot Air Oven

Feature	VPD Oven	Hot Air Oven

Drying Speed	Faster	Slower

Energy Efficiency	Higher	Lower

Drying Depth	Very deep (even internal insulation)	Surface and mid-layer drying

Equipment Cost	High	Medium to Low

Operation Complexity	Complex (needs trained operators)	Simple

Copper Oxidation Risk	None (vacuum environment)	Possible (due to open air)

Suitability	Large & power transformers	Small & medium transformers

🛡 Safety Measures

Monitor temperature and pressure sensors regularly

Use explosion-proof equipment when working with solvents

Ensure ventilation and solvent recovery systems are working

Follow standard drying time cycles as per transformer design

_______________________________________________________________________

CNC Plasma Cutting Machine – Detailed Working

🔹 What is a CNC Plasma Cutting Machine?

A CNC Plasma Cutting Machine is used to cut metal sheets (typically mild steel, stainless steel, aluminum) into precise shapes and profiles using a high-velocity jet of ionized plasma controlled via CNC (Computer Numerical Control).

It is widely used in transformer manufacturing to cut tank walls, base plates, covers, brackets, flanges, etc.

⚙ Working Principle

Plasma Generation

An electric arc forms between a nozzle and the workpiece, converting compressed gas (like air, nitrogen, argon) into plasma.

Metal Cutting

The hot plasma (10,000°C–30,000°C) melts the metal, and high-velocity gas blows away the molten material, creating a clean cut.

CNC Control

The cutting path is programmed via CAD/CAM software and executed by the CNC system for automated and precise cutting.

🛠 Key Components

Component	Function

Plasma torch	Generates and directs the plasma arc

CNC controller	Executes programmed cutting paths

Power supply	Generates high-frequency current for plasma arc

Gas supply unit	Provides compressed gas (air/N2/Ar)

Cutting table	Holds metal sheets in place

Exhaust system	Removes fumes and dust during cutting

📐 Key Parameters

Parameter	Value/Range

Cutting thickness	1 mm – 50 mm (depending on material)

Cutting speed	500 mm/min – 3000 mm/min

Accuracy	±0.2 mm to ±0.5 mm

Material types	MS, SS, Al, GI

✅ Advantages

High precision and speed

Cuts complex profiles and holes

Minimal edge finishing required

Less material waste

📘 2. Bending Machine – Detailed Working

🔹 What is a Bending Machine?

A Bending Machine (also called a Press Brake) is used to bend or form metal sheets into predefined angles or shapes, such as angles, channels, U-profiles, etc. It is essential in shaping transformer tanks, side walls, base frames, and enclosures.

⚙ Working Principle

The metal sheet is placed between a punch and a die.

The hydraulic press or mechanical ram pushes the punch down onto the metal, bending it into the die shape.

The bending angle is controlled by depth of punch movement and die shape.

🛠 Main Components

Component	Function

Upper punch	Applies force to bend the sheet

Lower die	Supports the sheet and determines bend angle

Back gauge	Positions the sheet for accurate bends

Hydraulic system	Drives the ram (punch movement)

Control panel	Sets bend angle, pressure, and stroke depth

📐 Key Parameters

Parameter	Typical Range

Bending capacity	1 mm – 20 mm sheet thickness

Bending angle range	0° – 180°

Material types	Mild steel, stainless steel, aluminum

Machine type	Hydraulic or mechanical

✅ Advantages

Consistent and repeatable bending

Can perform multiple bends on one part

High productivity and precision

Suitable for mass production of transformer parts

📘 3. Welding Machine – Detailed Working

🔹 What is a Welding Machine?

A Welding Machine is used to join metal parts (like transformer tank sides, covers, brackets, pipes) by melting and fusing the edges together, sometimes using filler material. Common methods include MIG (GMAW), TIG (GTAW), and Arc (SMAW) welding.

⚙ Working Principle

Electric arc is generated between the electrode and workpiece.

Heat from the arc melts the metal at the joint.

Filler metal may be added to strengthen the weld.

The molten pool solidifies to form a strong welded joint.

🛠 Main Components

Component	Function

Welding torch/gun	Directs arc and filler wire to weld zone

Power source	Supplies welding current and voltage

Ground clamp	Completes the electrical circuit

Gas supply (MIG/TIG)	Provides shielding gas (Argon/CO₂) to protect the weld

Filler wire/feed	Supplies metal for joint (MIG or TIG)

📐 Types of Welding Used in Transformer Fabrication

Welding Type	Description & Use

MIG Welding	Fast, semi-automatic welding used for tank walls and larger joints

TIG Welding	Precise, clean welding for thinner parts and stainless steel

Arc Welding	Common for structural frames and brackets (manual operation)

✅ Advantages

High strength joints

Custom shapes and assemblies possible

MIG/TIG offer clean, spatter-free finish

TIG gives better control for thin parts

❌ Common Issues & Precautions

Issue	Precaution

Warping due to heat	Use controlled heat input and clamps

Incomplete fusion	Proper current settings and electrode angle

Weld spatter	Use gas shielding and correct polarity

Joint misalignment	Use fixtures and templates

✅ Summary of Uses in Transformer Manufacturing

Machine Type	Main Applications

CNC Plasma Cutter	Cutting tank panels, cover plates, flanges, base plates

Bending Machine	Bending sheets into U channels, tanks, enclosures, mounting frames

Welding Machine	Joining transformer tank panels, frames, stiffeners, bushing pockets, pipes

Vacuum Oil Filling Machine – Detailed Working and Information

🔹 **What is a Vacuum Oil Filling Machine?

A Vacuum Oil Filling Machine** is a specialized system used to fill insulating oil (typically mineral or ester oil) into power and distribution transformers under vacuum conditions. The purpose is to remove trapped air/moisture from the windings and insulation and to completely impregnate the internal structure with dielectric oil, ensuring proper insulation and cooling of the transformer.

🎯 Objectives of Vacuum Oil Filling

Remove moisture and gases from insulation and windings

Ensure complete oil penetration into gaps and layers

Prevent air pockets, which can cause partial discharges or flashover

Improve the dielectric strength of the transformer

Ensure long-term reliability and performance

⚙ Working Principle of Vacuum Oil Filling Machine

The process works by creating a deep vacuum inside the transformer tank before and during oil filling. This low-pressure environment allows the oil to flow and impregnate windings deeply, without air resistance or trapped bubbles.

🔧 Step-by-Step Working Process

1. Preheating the Oil

Oil is filtered and heated to 50–60°C using an inline heater.

Heated oil has lower viscosity, improving flow and penetration.

2. Evacuation of Transformer Tank

The transformer tank is connected to the vacuum system.

A deep vacuum is drawn (up to 0.5–1 mbar) to remove air and moisture from windings and insulation.

Vacuum is maintained for several hours depending on transformer size.

3. Oil Filling Under Vacuum

With vacuum still applied, hot, filtered oil is filled from the bottom of the transformer.

Bottom filling avoids air entrapment and promotes uniform soaking of the coils.

The filling is done gradually to avoid stress on insulation.

4. Soaking Time

The transformer is kept under vacuum with oil filled to allow full absorption of oil into insulation layers.

5. Venting and Final Top-Up

Once filling is complete and soaking time has passed, the vacuum is slowly released.

Oil level is topped up to the required level before sealing the transformer.

🛠 Main Components of the Machine

Component	Function

Vacuum Pump System	Creates and maintains deep vacuum in transformer tank

Oil Heating Unit	Heats oil to reduce viscosity and enhance flow

Oil Filtration Unit	Removes moisture, gases, and particles from insulating oil

Oil Transfer Pump	Transfers oil from reservoir to transformer tank

Control Panel (PLC)	Manages vacuum level, temperature, pressure, flow rate, and alarms

Hoses/Connectors	Connect transformer to oil and vacuum systems

Moisture & Gas Indicators	Monitor oil quality before and after filling

📐 Typical Technical Parameters

Parameter	Typical Value

Operating Vacuum	0.1 – 1 mbar (very low pressure)

Oil Temperature	50°C – 65°C

Oil Flow Rate	500 – 5000 L/hr (depends on transformer size)

Filtration Accuracy	< 1 micron

Residual Moisture in Oil	< 10 ppm (post-filtration)

✅ Advantages of Vacuum Oil Filling

Ensures deep impregnation of insulating materials

Removes air pockets and moisture traces from core and coils

Enhances dielectric strength and thermal conductivity

Improves long-term reliability of transformers

Suitable for large power transformers, reactors, and bushings

❌ Common Challenges & Solutions

Issue	Solution

Incomplete vacuum or leaks	Check seals, hoses, and vacuum chamber integrity

Foaming or bubbling during filling	Fill slowly and monitor temperature/vacuum closely

Moisture not fully removed	Increase vacuum time or improve oil filtration

Overheating of oil	Use auto temperature controllers with alarm settings

🛡 Safety Measures

Ensure all valves and hoses are sealed before starting vacuum

Maintain temperature and pressure within limits

Use anti-static grounding to prevent spark during oil transfer

Never open transformer tank during vacuum filling

Use fire-resistant oil and explosion-proof motors where necessary

🏭 Applications in Transformer Manufacturing

Power Transformers (33 kV – 400 kV and above)

Distribution Transformers

Shunt Reactors

OLTC (On Load Tap Changer) chambers

HV Bushings and Insulated Devices

__________________________________________________________________________

High Voltage (HV) Testing Equipment – Detailed Information & Working

🔹 Purpose of HV Testing

High Voltage (HV) Testing is performed to:

Confirm the transformer insulation can withstand service voltage and overvoltages.

Detect any weak spots, internal faults, or partial discharges.

Ensure compliance with international standards (IEC, IS, ANSI).

⚙ Types of HV Tests Conducted

Test Name	Purpose

Power Frequency Withstand Test	Verifies insulation strength at rated frequency

Insulation Resistance Test	Measures resistance of insulation material

Leakage Current Test	Detects partial discharges or breakdown risk

Applied Voltage Test	Applies test voltage for 1 minute

🔧 Working Principle of HV Testing

A step-up transformer increases the input voltage to test level (e.g., 30 kV, 60 kV, 100 kV, etc.).

The test voltage is applied between:

HV winding and earth

HV to LV winding

LV winding and earth

The voltage is maintained for 1 minute (typically), and the leakage current is monitored.

If the insulation withstands the voltage without breakdown or flashover, the test is passed.

🛠 Main Components

Component	Function

HV Test Transformer	Steps up voltage to test level

Control Panel	Manages output, test time, current limits, safety shutoff

Capacitive Divider	Measures actual applied voltage accurately

Leakage Current Meter	Measures current leakage (sensitive in microamperes)

Safety Cage	Ensures operator safety during high voltage exposure

📐 Typical Parameters

Parameter	Value/Range

Output Voltage	10 kV – 300 kV

Test Duration	1 min to 5 min

Leakage Current	< 10 mA (varies by test object)

Frequency	50 Hz / 60 Hz

✅ Advantages

Non-destructive (in routine testing)

Reveals insulation degradation or manufacturing defects

Quick and reliable pass/fail assessment

Used for routine and type testing

❌ Safety Precautions

No one should touch the equipment during testing

Use proper earthing and interlocks

Test area should be enclosed with insulated floor and warning signs

Discharge stick must be used after test before touching the transformer

📘 2. Impulse Test Machine – Detailed Information & Working

🔹 What is Impulse Testing?

Impulse Testing is used to simulate lightning strikes or switching surges that may occur in service. It verifies the transformer’s ability to withstand transient overvoltages.

⚙ Working Principle of Impulse Test Machine

The system uses large capacitors charged to a high voltage (up to 1600 kV).

When discharged through a special wave shaping network (resistors & inductors), it creates a lightning impulse waveform:

e.g., 1.2 / 50 μs (rise time/fall time)

The impulse is applied between:

HV winding and earth

HV to LV winding

Or across windings for chopped wave tests

The resulting waveform is captured and compared to the expected impulse shape.

A failure is indicated by:

Waveform distortion

Flashover

Unusual oscillations or sudden drop in peak

🛠 Main Components

Component	Function

Impulse Generator	Produces high-voltage impulse by discharging capacitors

Charging Unit	Charges the capacitors to high voltage

Wave-shaping Network	Shapes the voltage pulse to simulate lightning/switching surge

Measuring System	Uses voltage dividers and oscilloscope to capture waveform

Test Controller Panel	Synchronizes charging, triggering, and measurement

📐 Impulse Wave Types

Type of Impulse	Application

Full Lightning Impulse	Simulates actual lightning (1.2/50 μs)

Chopped Lightning Impulse	Tests insulation near bushings

Switching Impulse	Used for UHV (Ultra High Voltage) units

📐 Typical Test Parameters

Parameter	Value/Range

Peak Voltage	100 kV – 1600 kV

Impulse Shape	1.2 μs rise / 50 μs fall (standard)

No. of Impulses	3 to 5 full waves + 2 chopped waves

Repetition Rate	1 impulse every 30–60 seconds

✅ Advantages

Accurately simulates real-world voltage surges

Detects weak points, especially in inter-turn insulation

Essential for type testing and special transformers

Captures failure points early to avoid costly field failure

❌ Precautions

Operate only by trained personnel

Keep the test area cleared and grounded

Use shielded rooms or faraday cages for sensitive instruments

Disconnect transformer from external circuitry

🏭 Use in Transformer Manufacturing

Machine	Stage of Use	Applies To

HV Testing Equipment	Final testing before dispatch	All transformers

Impulse Test Machine	Type testing / special transformers	Power transformers, HV distribution

__________________________________________________________________________

Spray Painting Booth & Powder Coating Booth

🛠 Why Painting or Coating is Done in Transformer Manufacturing?

Painting or powder coating serves the following purposes:

Corrosion resistance (especially for outdoor installations)

UV and weather protection

Electrical insulation in specific areas

Improved appearance and branding

Longer lifespan of transformer tanks and metal parts

📘 1. Spray Painting Booth – Detailed Working

🔹 What is a Spray Painting Booth?

A Spray Painting Booth is an enclosed area where liquid paint is sprayed onto metal parts like transformer tanks, base frames, and covers using compressed air spray guns. It includes ventilation, filtration, and overspray control.

⚙ Working Principle of Spray Painting Booth

Surface Preparation:

The transformer tank or part is cleaned, degreased, and shot/blast-cleaned.

Ensures paint adhesion.

Primer Application:

A corrosion-resistant primer (e.g., zinc-rich or epoxy) is applied.

Spray Coating:

A spray gun atomizes liquid paint using compressed air, forming a mist.

Paint is sprayed evenly over the surface inside the booth.

Drying/Curing:

Parts are moved to a drying area (air-dried or baked in an oven if required).

Some booths have built-in heating for faster drying.

Final Coat:

After primer, topcoat (PU/epoxy/enamel) is applied for finish and protection.

🧰 Main Components of Spray Booth

Component	Function

Spray gun	Atomizes paint using compressed air

Compressor	Supplies air to spray gun

Paint pump/system	Delivers liquid paint from container to gun

Enclosure walls	Prevent overspray and dust contamination

Air exhaust system	Pulls out paint mist and fumes

Filters	Capture overspray and dust particles

Lighting	Ensures clear visibility while spraying

📐 Technical Parameters

Parameter	Typical Range

Paint Type	Epoxy, enamel, PU

Air Pressure	4–6 bar

Film Thickness	80–120 microns (dry film)

Drying Time	2–12 hours depending on paint type

Airflow Type	Downdraft or crossdraft

✅ Advantages of Spray Painting

Uniform finish with minimal surface defects

Works on complex shapes and edges

Can be used for large parts like full transformer tanks

Easier for touch-up and field repair

❌ Disadvantages

Paint contains solvents and VOCs (environmental concern)

Requires longer drying/curing time

Manual spray skill affects finish quality

📘 2. Powder Coating Booth – Detailed Working

🔹 What is a Powder Coating Booth?

A Powder Coating Booth is used to apply electrostatically charged powder paint (no liquid/solvent) onto grounded metal parts. After coating, parts are baked in an oven, where the powder melts and cures into a hard, protective film.

It is used in transformer manufacturing for:

Radiators

Control boxes

Terminal covers

LV & HV metal accessories

⚙ Working Principle of Powder Coating

Surface Cleaning:

Parts are cleaned using phosphate wash or sand blasting.

Powder Spray Application:

Powder paint particles are negatively charged in the gun.

The metal part is electrically grounded.

Electrostatic attraction causes powder to stick to the surface.

Curing in Oven:

The coated part is baked at 160–220°C for 15–30 minutes.

Powder melts, flows, and forms a solid, durable coating.

🧰 Main Components of Powder Coating Booth

Component	Function

Electrostatic spray gun	Charges powder particles and sprays them

Powder hopper	Stores and delivers powder to gun

Control panel	Adjusts voltage, powder feed rate, and spray settings

Booth enclosure	Contains overspray

Recovery unit	Recovers unused powder for reuse

Curing oven	Bakes the part to cure the powder coating

📐 Technical Parameters

Parameter	Typical Range

Voltage at gun	30–100 kV

Coating Thickness	60–100 microns

Curing Temperature	160–220°C

Curing Time	10–30 minutes

Powder Types	Epoxy, polyester, hybrid

✅ Advantages of Powder Coating

No solvents, environmentally friendly

Hard, durable finish resistant to scratches, corrosion, and UV

High transfer efficiency (up to 98% with recovery system)

Minimal overspray wastage

Available in various colors and textures

❌ Disadvantages

Requires high-temperature oven for curing

Difficult to coat inside corners or hollow areas

Not suitable for non-metallic parts

Color change requires complete gun and booth cleaning

📊 Spray Painting vs Powder Coating – Comparison

Feature	Spray Painting	Powder Coating

Paint Type	Liquid (solvent-based)	Dry powder (electrostatic)

Drying Method	Air dry or heat	Curing in oven

Film Hardness	Medium	Very Hard

Environmental Impact	High VOCs	Eco-friendly (no VOCs)

Suitable for Large Parts	Yes (like tanks)	Limited (depends on oven size)

Color Flexibility	High	Medium (requires full cleaning to change)

Durability	Moderate to High	Very High

🛡 Safety Measures

Use PPE: respirator mask, gloves, eye protection

Spray booths must have exhaust and filtration systems

No open flames or sparks near powder or paint booths

Handle powders with static grounding to prevent fire risk

Maintain booth cleanliness to avoid contamination